Proof of Concept (POC) Document

Tender for the Provision of a System for the Introduction of Modern Technologies and Artificial Intelligence in Natural Reserves (Phase I: Ras Al Shajar Natural Reserve)

- Purpose and Objectives

This document aims to establish a comprehensive framework for the Proof of Concept (POC) of the technical solutions proposed by Ankaa Space & Technologies L.L.C for the project "Provision of a System for the Introduction of Modern Technologies and Artificial Intelligence in Natural Reserves (Phase I: Ras Al Shajar Natural Reserve)." The test will be conducted on a limited scale that represents the full project environment, focusing on the key technical components. The main objective is to practically and realistically verify the efficiency of the proposed smart systems in enhancing environmental monitoring operations and protecting wildlife.

- Specific Objectives of this Test:

- To confirm that all system components (cameras, BrainBox Edge processing unit, PTZ thermal camera tower, DJI Dock 2, and a temporary cloud/local platform Oman BioWatch) work seamlessly and reliably as an integrated system.

- To demonstrate the system's ability to operate effectively in the actual environmental conditions of the reserve.

- To measure the performance of AI-powered detection technologies and the system's responsiveness to alerts.

- Scope of the Test

- Location: The POC will be conducted at a designated "choke point" within the Ras Al Shajar Reserve, such as a valley or a natural wildlife corridor. This location has been selected to simulate the most challenging scenarios.

- Duration: The test period will be for 60 days, the period will start from the date the system components are fully configured and put in place.

- Equipments: A subset of the main system components will be deployed for the POC:

- Trap Cameras: They will be deployed at strategic points within the test area.

- BrainBox Edge AI Device: It will be installed on-site to process the data from the cameras locally.

- DJI Dock 2 Drone Docking Station: It will be placed in a secure location near the test point to conduct automated flight missions.

- PTZ Camera: It will be tested on-site to demonstrate its thermal capabilities.

- Oman BioWatch Dashboard: It will be set up in the designated monitoring room for the purpose of the test.

- Support and response time: the company should demonstrate how technical support will be delivered throughout the period of POC. Instant response to reported system failures will be one of the main measurements of the success of the POC.

*Installing any equipment inside or outside the range and parameters of the reserve will always require formal approval from EA. The company should advise with the recommended locations for any hardware installation, EA will study the feasibility of the recommended places and grant permission if applicable.

- Success Criteria & Key Performance Indicators (KPIs)

The success of the POC will be evaluated based on the following measurable criteria:

a. Evaluation of Wireless Network Stability (LoRaWAN/4G): Assess the stability of wireless networks within the protected area.

b. Detection Accuracy: Smart cameras must demonstrate the ability to detect and classify targets (human, animal, vehicle) with an accuracy of at least 85%. This will be verified through manual tests.

c. Alert Response Time: An alert must be sent to the Oman BioWatch platform within less than two minutes from the moment a target is detected.

d. System Availability: The system must operate continuously without interruptions or major failures.

e. Automated Drone Operation: The drone must be able to take off automatically upon receiving a command from the platform, reach the test location, stream live video, and then return and land successfully at the docking station.

f. Thermal Capability of the PTZ Camera: The thermal camera must demonstrate the ability to detect targets (person or vehicle) in complete darkness or fog at distances of up to 3 km.

g. AI Capability Test: The AI must reduce false alarms to less than 10%.

h. Efficiency of the operations and control center, and the basic requirements for uninterrupted operations.

i. Ensuring the presence of a technical support team for backup and technical assistance in case of emergency faults or permanent damages requiring direct and immediate intervention 24/7.

| Rank | Success Criterion / KPI | Measurable Target | How to Measure Success | Verification Method | Rating (1–5) |
| --- | --- | --- | --- | --- | --- |
| 1 | Detection Accuracy | ≥ 85% accuracy (human / animal / vehicle) | 1. Run 100 scripted test events (33 human, 33 animal, 34 vehicle). 2. Ground-truth each event manually (video review + GPS tag). 3. Calculate True Positives / (TP + FP + FN). 4. Success = ≥ 85%. | Manual audit log + AI classification report | 5 |
| 2 | Alert Response Time | < 2 minutes from detection → Oman BioWatch | 1. Timestamp T1 = AI detection event. 2. Timestamp T2 = Alert received in BioWatch UI. 3. Measure T2 – T1 for 50 random alerts. 4. Success = 95th percentile < 120 sec. | Automated timestamp logs + BioWatch API receipt | 5 |
| 3 | AI Capability Test (False Alarms) | < 10% false alarm rate | 1. Run 200 non-threat scenarios (birds, wind, shadows, etc.). 2. Count AI-triggered alerts. 3. Success = ≤ 20 false alarms (10%). | Controlled field log + false-positive review panel | 5 |
| 4 | Wireless Network Stability | ≤ 5% packet loss over 24 h; seamless failover | 1. Send ping every 10 sec on LoRaWAN & 4G. 2. Log packet loss % per hour. 3. Trigger 3 forced failovers (cut primary link). 4. Success = ≤ 5% loss + < 30 sec switch. | Network monitor (PRTG/Zabbix) + failover log | 4 |
| 5 | System Availability | 99.9% uptime (no failure > 5 min) | 1. Run 7-day continuous test. 2. Auto-log downtime events. 3. Success = total downtime ≤ 10 min. | Health-check dashboard + incident log | 4 |
| 6 | Automated Drone Operation | Full autonomous cycle | 1. Trigger 10 missions from BioWatch. 2. Checklist: Takeoff → GPS waypoint → 2 min live stream → RTB → Dock. 3. Success = 10/10 complete without manual input. | Drone flight log + video stream record | 3 |
| 7 | Thermal PTZ Camera Capability | Detect person/vehicle @ 3 km in zero visibility | 1. Place human & vehicle targets at 3 km (GPS verified). 2. Test in total darkness & artificial fog (visibility < 50 m). 3. Success = AI flags target + operator confirms on thermal feed. | Night/fog chamber + field range test | 3 |
| 8 | Efficiency of Operations & Control Center | SOP response < 5 min; 24/7 staffing | 1. Run 5 simulated incidents. 2. Measure alert → operator action time. 3. Verify shift roster (24/7 coverage). 4. Success = all < 5 min + roster signed. | Incident drill timer + duty roster | 2 |
| 9 | Technical Support Team (24/7) | < 1 h on-site response; spare parts ready | 1. Trigger 3 mock failures (random times). 2. Start timer at call → technician arrival. 3. Check spare parts inventory (list signed). 4. Success = all < 60 min + parts available. | SLA response log + inventory checklist | 2 |

Expected Deliverables

Upon the successful completion of the POC, a certified technical report including a visual presentation will be delivered, including:

- Success and failure rates, and system reliability.

- Assessment of power consumption and operational sustainability.

- False alarm rate.

- An official stakeholder review and results presentation session.

- Recommendations for system development.

- Protection and Compliance Mechanisms

- All data must be encrypted, and access must be monitored with multi-level permissions.

- Monitoring tools and dashboards should be included within the POC and connected to EA head office.

- All resulting codes, images, and data become the property of the Environment Authority.

- The Authority has the right to conduct a security or technical audit during the trial.

- EA Obligations

The Environment Authority:

  - Facilitate permits and site access.

  - Provide contact points for coordination.

  - Approve final reports and provide feedback.

  - Provide qualified personnel to be the main point of contact for the POC.

- Risk Management

- The company should provide a full report of the risks and challenges encountered during the POC with their mitigation plan.

- Early Termination Conditions

- The environment authority has the right to cancel the trial in case of:

  - The company’s failure to comply with essential specifications.

  - Emergence of unacceptable security risks.

  - Unjustified delays in implementation.

------------------------------------------------------------------------------ End of Document